THE GEORGE WASHINGTON UNIVERSITY

DEPARTMENT OF ELECTRICAL AND COMPUTER ENGINEERING 
	ECE 122-FALL 2007

FINAL PROJECT-OPTION 3

VENDING-MACHINE CONTROL UNIT

The customers of   KA*Tech Corporation   were quite impressed by your performance on the multi-function register project. Consequently, the company was awarded a new project to design, implement and document a prototype circuit that will control the operation of vending machines. In order to control costs, the customer requires the control circuit to be implemented in a single chip. The 
only external components allowed are discrete resistors and capacitors. This control unit must 
interface with three auxiliary units: (i) the coin-input  and drink selection unit, (ii) the change-
dispensing unit, and (iii) the drink-dispensing unit, as shown in the following block-diagram:

DT	2

ND

Coin

input and

drink	DD

selection

unit

QD	Vending-machine

control unit

CR

NC

DC

QC

DR1

DR2

CC

Change

dispensing 
	unit

Drink

dispensing 
	unit

Inputs to control unit	Outputs from control unit

ND = Nickel deposited	CC = Collect coins

DD = Dime deposited	NC = Nickel change

QD = Quarter deposited	DC = Dime change

CR = Coin return	QC = Quarter change

DT1 = Drink type 1	DR1 = Dispense drink type 1

DT2 = Drink type 2	DR2 = Dispense drink type 2

Although the operation of a vending machine is familiar, let us review a typical transaction. A

series  of coins, restricted to nickels, dimes, or quarters is inserted one at a time into the coin

GWU/SEAS/ECE 122: Fall 2000; Prepared by: M. Afridi and C. E. Korman

return button  activates (VH)

input unit. This unit holds them in a temporary location and produces a pulse on one of its output lines indicating the value of each coin inserted. The vending machine offers two choices of drinks. The first drink, DT1, costs sixty cents and the second drink, DT2, costs eighty-five cents. 
The buyer may push the coin-return button at any time before the release of a drink, generating a 
signal to the controller indicating that all money has been returned. The buyer may press the 
appropriate drink type at any given time. If enough money was deposited into the machine then 
the appropriate drink and change is released.  Obviously, the controller must generate output 
signals to the drink and change dispensing units. In addition, once a drink is released, a signal is 
generated causing the coin-input unit to transfer all coins from the temporary location to the 
permanent location.

Auxiliary-unit input-output signal specifications:

Coin - input and drink	Change dispensing unit	Drink dispensing unit

selection unit

ND, DD, QD signals: unit	This unit has three input	This unit has two inputs

will accept nickels, dimes	lines: one for nickels (NC),	(DR1, DR2) and dispenses

or quarters and produce a

200ms pulse (VH) on line corresponding to that coin type; Outputs will be

inactive (VL) for at least
500ms between successive 
pulses.

CR signal: pushing the coin

the coin return line as long as button is pushed.

CC signal: a 200ms pulse (VH) on this line causes deposited coins to be

collected.

DT1, DT2 signals: a 200ms pulse is generated (VH) to  indicate the drink type.

one for dimes (DC) and one	one drink for each input

for quarters (QC); one coin	pulse (VH) lasting longer

of appropriate value is	than 200ms.

released for each input

pulse (VH) lasting longer

than 200ms; inputs must be

inactive (VL ) for at least

200ms between pulses.

It is your responsibility to make sure that the above pulses are generated and the timing requirements are met. The only input one may assume from the user is an arbitrary pulse trigger. You must implement this circuit in 0.8 um HP CMOS n-well technology. You may assume that a 5 VDC power supply is available. In your documentation you must clearly demonstrate the correct operation of this circuit for at least five different test cases (coin input sequence and drink type).  The project is due December 7th, 2007.

GWU/SEAS/ECE 122: Fall 2000; Prepared by: M. Afridi and C. E. Korman