The George Washington University 
School of Engineering and Applied Science 
Department of Electrical and Computer Engineering 
ECE 20 – SPICE Tutorial 6

Designing a Common-Collector Amplifier

Background:

In the previous lab you design a common-emitter (CE) amplifier.  Voltage gain (Av) is easy to achieve with this type of amplifier.  As you discovered the input impedance (RIN) of the CE is moderate-to-high (on the order of a few kilo-Ohms).  The output impedance (ROUT) is high (roughly the value of RC).  This makes the common-emitter amplifier a poor choice for ‘driving’ small loads.

|  | RIN ROUT |
| --- | --- |
| Fig 1.1 – Common Collector Amplifier | Fig 1.2 - Small Signal (AC) equivalent (neglecting Rsig) |

A common-collector (CC) amplifier typically has a high input impedance (typically in the hundred kilo-Ohm range) and a very low output impedance (from 1 to ~tens of Ohms).  This makes the common-collector amplifier excellent for ‘driving’ small loads.  As you discovered in lab 5, the common-collector amplifier has a voltage gain of about 1, or unity.  The common-collector amplifier is considered a voltage-buffer, as the voltage gain is unity, the voltage signal applied at the input will be duplicated at the output; for this reason the common-collector amplifier is typically called a “emitter-follow amplifier.”  The common-collector amplifier can be thought of as a current amplifier.

When the common-emitter amplifier is cascaded to a common-collector amplifier, the CC can be thought of as an ‘impedance transformer.’  It can take the high output impedance of the CE amplifier and ‘transform it’ to a low output impedance capable of driving small loads.

Figure 1.1 shows a typical configuration for a common-collector amplifier.  The input voltage is applied to the base while the output voltage is measured at the emitter.

From the AC equivalent of the common-collector amplifier in figure 1.2, we can derive the input impedance, output impedance, and voltage gain:

RIN   = R1 ||  R2 || [βre + (β+1) RE]    (no load)  (Remember: re = VT / IE, where VT = 26mV

ROUT = RE || re  but  re << RE  → Zo ≈ re   (notice this is VERY small)

Av=VoutVin=β+1ib∗RE(β)(ib)(re)+(β+1)(ib)(RE)≃βib∗RE(β)ib(re+RE)=RE(re+RE), but since re << RE, then:

Av≈1

Designing a Common-Collector Amplifier

Problem: Design a Common-Collector Amplifier using the 2N3904 transistor that meets the following specifications:

IC = 1mA

VCC = 20 Volts

Rin = 70KΩ

RL = 510Ω

vin = 10mV @ 10kHz

Step 1) Determine the size of RE

- We typically make VE = ½ VCC, to ensure the largest possible symmetric output voltage swing (around VE)

- It is safe to assume that IE ≈ IC

- Calculate the value of RE

Step 2) Determine the “Q” point of the transistor

- Because you now know VCE & IC, you can use the same procedure from the “Designing a Common-Emitter Amplifier Tutorial” to create an IV-curve for the transistor & determine the Q-point of the transistor.  This will help you determine the necessary “base current” needed to achieve the specified IC.

- Use the Q-point data to find DC values for: IB, VB, IE, β

Step 3) Use VCC, VB, IB, IE, and RIN, to find R1 and R2

- Follow the procedure from the “Designing a Common-Emitter Amplifier Tutorial” to generate the same 3 equations for VBB (eqn 1); RB (eqn 2); and IB (eqn 3).

- Use the equation derived the first part of this tutorial for RIN as equation 4.

- Calculate R1 and R2 using the 4 equations

Step 4) Check your calculations

- Using the RIN equation, calculate RIN.  Is it 70k?

- Using the ROUT equation, calculate ROUT, is it very small?

Step 5) Set values for CC1 & CC2

- The impedance of a capacitor Zc = 1/j2πfC, make CC1, CC2 look like a ‘short’ at 10kHz (the input frequency), and make sure the size you choose for CC1 and CC2 matches a capacitor value you have in your ECE 20 kit.

Step 6) Determine Current Gain (Ai) for the amplifier

- Current gain is defined as: Ai= Iout / Iin

- Use the equations for Av, RIN, and ROUT, ohms law & a little algebra to determine the equation for Ai

- Calculate Ai for your amplifier, verify current gain using a SPICE transient simulation.

Step 7) Verify your calculations using SPICE

- Simulate your amplifier.  Check the bias-point analysis to determine if your transistor is at the Q-point you desire.

- Perform a transient simulation to very the Voltage gain, current gain, RIN and ROUT.