The George Washington University 
 School of Engineering and Applied Science 
 Department of Electrical and Computer Engineering 
 ECE 20 - LAB 
Midterm Project – DC Power Supply

### Objectives:

Your goal is to simulate, design, build and demonstrate a practical DC power supply that can be used as the power supply of various consumer electronic devices. Your power supply must meet or exceed following specifications:

### Prelab (due at the beginning of the demonstration lab)

Complete the following before coming to the lab and submit a copy to the instructor. Please make sure to keep a copy for your reference.

- Complete circuit diagram drawn on PSPICE

- Brief working principle of the circuit. (Show your technical understanding of the circuit here)

- PSPICE simulation

### Design Specifications:

- Your power supply will have an on state indicator (LED)

- Input Voltage: 120V rms @ 60Hz

- Voltage Output: Two regulated outputs of 12V and -12V respectively

- Load regulation: Less than 2%

  - Assume 100 ohm full load. (You can connect multiple resistors in parallel , so that none of the resistors dissipates more power than its maximum power limit.)

- Instead of resistors in parallel a ‘power resistor’ may be used

  - Note, GTA may set the load to 10 Ohms, to allow midterm project to support the load of the final project – 10 WRMS Amplifier

  - An inductor may need to be inserted in design to meet ripple requirements

- Ripple: Less than 1%

### Demonstration (60%):

You have to demonstrate that your circuit meets all the specification to your GTA

(see grading criteria on the next page to see how the demonstration will be graded)

### Report (40%) (due the week after the demonstration):

Write a brief report (using the lab report format) which will include following:

- Block Diagram of major stages

- Circuit Diagram

- PSPICE simulation

- Circuit Theory and operation

- Test results meeting the specifications

The George Washington University

School of Engineering and Applied Science

Department of Electrical and Computer Engineering

ECE 20 - ENGINEERING ELECTRONICS

Midterm Project Grading Sheet

Student’s Name: _______________________________

===SETUP (45 pts total)===

(student must have each item with circuit to show TA during demonstration)

Schematic + Hand calculations must contain:

____(2.5pts) Schematic

____(5pts)    Ripple calculation, correctly determining size of filter capacitor

____(5pts)    Load resistor calculation, accounting for individual power dissipation of resistor

____(2.5pts) Load regulation calculation

____(5pts)    LED’s resistor size calculation

____(5pts)    Current draw of each branch calculated, meets regulator spec

____(5pts)    Power dissipation of all components after regulator calculated

____(2.5pts) Neatness

____(5pts)    Spice Simulation w/graphs

____(2.5pts) Proper Calibration of Oscilloscope

(Channel 1 on source voltage, Channel 2 on output signal, lose points for auto-set)

____(5pts)    Student is able to explain operation of circuit

===CIRCUIT DEMONSTRATION (55 pts total)===

(student is to perform all measurements – show to TA)

____(5pts)    LED indicator lights up

____(5pts)    DMM measures 12VDC across load

____(5pts)    DMM measures -12VDC across load

____(5pts)    Proper Output of Voltage Regulator on Scope (12 VDC overlaid onto Channel 1)

____(2.5pts) Ripple < 1%

____(2.5pts) Load Regulation < 2%

____(5pts)    Proper Output of Filter Capacitor on Scope

____(5pts)    Proper Output of Rectifier on Scope (May need to pull filter cap)

____(2.5pts) Proper Load Resistance = 100 Ohms (or 16 ohms)

____(5pts)    Power Dissipated by load = 1.44 Watts (measure current to show) (or 9 Watts)

____(5pts)    Power dissipated by each load resistor below tolerance limit of resistor (require

measurement of current to verify)

____(5pts)    Current flowing in LED branch ~20mA

____(2.5pts) Voltage across LED < 3V

TOTAL: ______ (100pts)