Optimizing Range and Height of a  Projectile using Matlab

S. Ahmadi and K. Bulusu

Problem Statement:

The problem considered is about idealized projectile motion, without any air drag,  indicated in the figure with a dashed line. A gun is being test fired in a firing range to estimate the distance a cannon ball travel or in other words, estimate it’s range (R). The initial velocity of the cannon is vo, which is shot at angle θ.

The range ( R) is a function of  initial velocity of the cannon (vo) and the angle at which it is shot (θ), with the following relation:

R = (vo)2 sin(2θ)/g

Where ‘g’is the acceleration due to gravity = 9.8 m/s.

The height the cannon reaches ( H) is also a function of  initial velocity of the cannon (vo) and the angle at which it is shot (θ) with the following relation:

H = (vo)2 sin2(θ)/2g

R

H

θ

vo

MATLAB Project:

Write a matlab script with the following requirements:

- Vary the angle θ between 10 degrees and 90 degrees in steps of 2 degrees and calculate the range ( R ) for an initial velocity of the cannon (vo = 5 m/s)

(note: is matlab using degrees or radians?  Check help to find out.)

- Plot R vs. θ for vo = 5 (indicate units on axis or legend)

- Find the maximum range (R) from the data generated.

- Find the value of (θ) –in degrees-- that yields the maximum value of (R).

Note: for questions 3 & 4, matlab commands must be used to obtain these values, reading the value off of the graph is unacceptable

- Repeat questions 1-4 by calculating H and plot H vs. θ for vo = 5m/s